# Post Office Protocol Version 3 (POP3) - Comprehensive Technical Report

## 1. Introduction

Post Office Protocol Version 3 (POP3) is an application-layer email retrieval protocol defined in RFC 1939. It enables email clients to retrieve electronic mail messages from a remote mail server using a TCP connection.

Default Port: 110 (Unencrypted)

Secure Port: 995 (POP3 over SSL/TLS)

OSI Layer: Application Layer (Layer 7)

Transport Protocol: TCP

## 2. Objectives and Design Philosophy

POP3 was designed during an era of limited connectivity and expensive server storage. Its core objective is to download email messages from a server to a local client, allowing users to read and manage emails offline.

## 3. Architecture Overview

POP3 follows a client-server architecture. The client initiates a TCP connection to the mail server, authenticates, retrieves messages, and optionally deletes them from the server.

## 4. Working Methodology

### 4.1 Authorization State

Upon connection, the server sends a greeting. The client authenticates using USER and PASS commands or optionally APOP authentication.

### 4.2 Transaction State

After successful authentication, the client may issue commands such as LIST, RETR, DELE, STAT, NOOP, and RSET to manage mailbox content.

### 4.3 Update State

Triggered by the QUIT command. Messages marked for deletion are permanently removed from the server and the connection is terminated.

## 5. Common POP3 Commands

USER – Specifies username

PASS – Specifies password

STAT – Displays mailbox statistics

LIST – Lists messages

RETR – Retrieves a message

DELE – Marks a message for deletion

NOOP – No operation (keep-alive)

RSET – Resets deletion marks

QUIT – Ends session

## 6. Security Mechanisms

Originally, POP3 transmitted credentials in plaintext. Modern implementations use SSL/TLS encryption (POP3S on port 995) or STARTTLS to secure communications.

APOP authentication uses MD5 hashing, but it is considered weak by modern cryptographic standards.

## 7. Attack History and Exploitation Patterns

• Cleartext credential sniffing on port 110

• Brute-force and dictionary attacks

• Credential stuffing using leaked passwords

• Buffer overflow vulnerabilities in early server implementations

• Malware harvesting stored POP3 credentials

• STARTTLS downgrade attacks

## 8. Advantages

• Simple and lightweight protocol

• Efficient for low-bandwidth environments

• Suitable for single-device offline usage

• Minimal server storage requirement

## 9. Limitations

• No multi-device synchronization

• No server-side folder management

• No advanced message flag support

• Limited scalability for enterprise systems

• Security risks if TLS is not enforced

## 10. POP3 vs IMAP Comparison

POP3: Downloads and optionally deletes emails; optimized for offline access.

IMAP: Synchronizes emails across devices; maintains server-side state and folders.

## 11. Modern Relevance

Although largely replaced by IMAP and webmail systems, POP3 remains in use for legacy systems, small organizations, and environments requiring offline-first access.

## 12. Conclusion

POP3 remains a foundational email retrieval protocol with a simple and efficient design. However, its limitations in synchronization and security make it less suitable for modern multi-device environments. Secure deployment using SSL/TLS and strong authentication mechanisms is essential.